昌 葧 有 限 公 司

Dr.POWER CORPORATION

零 件 規 格 書/承 認 書

##### SPECIFICATION FOR APPROVAL

CUSTOMER :            金阿波羅股份有限公司

DESCRIPTION :             7φ*16mm,1.3V

MODEL :                      FD-7BL5

CUSTOMER PART NO :

APPROVED SIGNATURES

| 臺北市北投區吉利街131號3樓 ### Rev | Date | #### Description | Designed | Checked | Approved |
| --- | --- | --- | --- | --- | --- |
| A | 2010/10/05 | Release |  | Ken |  |

3F, NO. 131, JILI ST., PEITOU, TAIPEI, TAIWAN, R.O.C.

TEL : 886-2-2820-7419． FAX : 886-2-2827-0348

| SPECIFICATION |  | 技術指導書 |  |  |
| --- | --- | --- | --- | --- |
| VIBRATION MOTOR | MODEL NO.: # FD-7BL5 |  | DATE: 2010/10/05 |  |
| 振動馬達 |  |  | PAGE: 1/6 | REV: A |

- CONSTRUCTION FORM 結構形式:

| NO. 序號 | ITEM 項目 | CONTENTS 內容 |
| --- | --- | --- |
| 1-1 | MOTOR CONSTRUCTION 馬達結構 | 7 CYLINDRICAL DC CORELESS MOTOR 直徑為7mm的空心杯振動馬達 |
| 1-2 | NUMBER OF PHASES 相數 | 3-PHASE (CONNECTION) 三相 |
| 1-3 | NUMBER OF MAGNET POLES 磁綱極數 | 2-POLES (INNER MAGNET TYPE) 兩極 (磁綱內置) |
| 1-4 | COIL CONSTRUCTION 線圈結構 | CYLINDRICAL MOVING COIL 圓柱型線圈 |
| 1-5 | RECTIFYING METHOD 換向方式 | PRECIOUS BRUSH AND COMMUTATOR 電刷換向器裝置 |

- STANDARD OPERATING CONDITION 標準運行條件:

| NO. 序號 | ITEM 編號 | SPECIFICATION 規格 |
| --- | --- | --- |
| 2-1 | RATED VOLTAGE 額定電壓 | DC 1.3V |
| 2-2 | RETED LOAD 額定負荷 | VIBRATION WEIGHT (AS SPECIFIED IN OUTLINE DRAWING) 偏心鍾(尺寸參考相關圖紙) |
| 2-3 | ROTATION DIRECTION 回轉方向 | CW(CLOCKWISE) FROM THE COUNTER WEIGHT 順時針方向 |
| 2-4 | MOTOR POSITION 擺放位置 | ANY POSITION IS AVAILABLE 任何位置都可以 |
| 2-5 | VOLTAGE RANGE FOR USE 工作電壓範圍 | DC 0.9~1.6V |
| 2-6 | WORKING TEMPERATURE RANGE 工作溫度範圍 | -20℃~60℃ 10~90% (RELATIVE HUMIDITY/相對濕度) |

| SPECIFICATION |  | 技術指導書 |  |  |
| --- | --- | --- | --- | --- |
| VIBRATION MOTOR | MODEL NO.: # FD-7BL5 |  | DATE: 2010/10/05 |  |
| 振動馬達 |  |  | PAGE: 2/6 | REV: A |

3.ELECTRICAL CHARACTERISTICS 電氣性能:

| NO. 序號 | ITEM 編號 | MEASURING CONDITION 測試條件 | SPECIFICATION 制品規格 |
| --- | --- | --- | --- |
| 3-1 | RATED SPEED 額定轉速 | AT RATED VOLTAGE AND RATED LOAD 額定電壓及負荷下測試0 | 5500±1500 rpm |
| 3-2 | RATED CURRENT 額定電流 | AT RATED VOLTAGE AND RATED LOAD 額定電壓及負荷下測試 | 105 mA max |
| 3-3 | STALL CURRENT 堵轉電流 | AT RATED VOLTAGE AND SHAFT LOCK 額定電壓及鎖定電機軸條件下測試 | 130mA max |
| 3-4 | STARTINGVOLTAGE 起動電壓 | AT RATED LOAD AND STEP VOLTAGE 額定負荷及電壓直投法條件下測試 | 0.9V max |
| 3-5 | STARTING TORQUE 起動扭矩 | AT RATED VOLTAGE AND SHAFT LOCK 額定電壓及鎖定電機軸條件下測試 | 0.4G*CM min |
| 3-6 | TERMINAL RESISTANCE 電樞電阻 | MEASURE BETWEEN TERMINALS 端子間測量 | 10Ω±20% |
| 3-7 | INSULATION RESISTANCE 絕緣電組 | 100V DC BETWEEN LEAD WIRE AND CASE 在導線與馬達機身之間提供100V DC | 10 MΩmin |
| 3-8 10 cm Test Block Motor Microphone Sound Level Meter | MECHANICAL NOISE 機械噪音 | AT RATED VOLTAGE AND RATED LOAD BACK GROUND NOISE: 28 dB (A) MAX 額定電壓及負荷下測試背景噪音不超過28分貝 | 50dB(A) max |

ADD: MEASURING CONDITION 測試條件

1.: MEASURING TEMPERATURE: 10-35 ℃ 測試溫度: 10-35 ℃

2.: MEASURING RELATIVE HUMIDITY: 45% - 75% 測試相對濕度: 45% - 75%

| SPECIFICATION |  | 技術指導書 |  |  |
| --- | --- | --- | --- | --- |
| VIBRATION MOTOR | MODEL NO.: # FD-7BL5 |  | DATE: 2010/10/05 |  |
| 振動馬達 |  |  | PAGE: 3/6 | REV: A |

4. MECHANICAL CHARACTERISTICS 機械特徵:

| NO. 序號 | ITEM 項目 | MEASURING CONDITION 測試條件 | SPECIFICATION 規格 |
| --- | --- | --- | --- |
| 4-1 | CONFIGURATION 外觀尺寸 | VERNIER CALIPER 油標卡尺 | AS SPECIFIED IN OUTLINE DRAWING 參考相關外形圖紙 |
| 4-2 | APPEARANCE 外觀 | VISUAL EXAMINATION 目測 | SHOULD BE NO EVIDENCE OF MECHANICAL DAMAGE, CRACK, RUST, AND STAIN 不得有明顯的機械擦痕, 裂縫, 銹跡及污點 |
| 4-3 | WEIGHT 重量 | BALANCE 天平 | 3.1±0.3G (APPROX) 3.1±0.3克 (左右) |
| 4-4 | SHAFT END PLAY 軸向間隙 | DISTANCE OF AXIAL PLAY ON SHAFT 軸向間隙 | 0.3mm max |

5. RELIABILITY TEST 可靠性試驗:

SHOULD BE SATISFIED WITH THE REQUIREMENTS OF TABLE 1 AFTER

TEST. 通過以下測試後應滿足表1的規格需求.

TABLE 1: REQUIREMENTS ITEM 表一: 評估項

| NO. 序號 | ITEM 項目 | SPECIFICATION 規格 |
| --- | --- | --- |
| 5-1 | RATED CURRENT 額定電流 | INITIAL DATA: ±30% MAX 最大不超過初始始值的±30% |
| 5-2 | RATED SPEED 額定轉速 | INITIAL DATA: ±30% MAX 最大不超過初始始值的±30% |
| 5-3 | STARTING VOLTAGE 起動電壓 | 1.0V DC MAX 最大不超過1.0V DC |
| 5-4 | OTHERS 其它 | - SHOULD NOT FALL IN STARTING - SHOULD NOT BE UNUSUAL MECHANICAL NOISE - 正常啟動 - 無異常噪音 |

| SPECIFICATION |  | 技術指導書 |  |  |
| --- | --- | --- | --- | --- |
| VIBRATION MOTOR | MODEL NO.: # FD-7BL5 |  | DATE: 2010/10/05 |  |
| 振動馬達 |  |  | PAGE: 4/6 | REV: A |

6. RELIABILITY TEST ITEMS可靠性測試項目

| NO. 序號 | ITEM 項目 | TEST CONDITION 測試條件 |
| --- | --- | --- |
| 6-1 | LIFE TEST 壽命測試 | - VOLTAGE: 1.3V 測試電壓: 1.3伏 - LOAD:DOUNTER WEIGHT 負荷:偏心鍾 - TEMPERATURE:20±2℃溫度: 20±2℃ - RELATIVE HUMIDITY: 65±5% 相對濕度: 65±5% 秒 TEST MODE: 20,000 TIMES AFTER OPERATION CONDITIONS AS SEC. ON 8 SEC. OFF 1 SEC. 測試方式: 開8秒, 關1秒, 循環20, 000次 |
| 6-2 | HIGH EMPERATURE EXPOSURE 高溫放置 | EXPOSURE TO 80℃ FOR 96 HOURS THEN RETURN TO ORDINARY TEMPERATURE AND HUMIDITY FOR 2 HOURS. 在溫度80℃條件下放置96小時後在正常溫度和濕度條件下放置2小時 |
| 6-3 | LOW TEMPERATURE EXPORSURE 低溫放置 | EXPOSURE TO -40℃ FOR 96 HOURS, THEN RETURN TO ORDINARY TEMPERATURE AND HUMIDITY FOR 2 HOURS. 在溫度-40℃條件下放置96小時後在正常溫度和濕度條件下放置2小時 |
| 6-4 | HUMIDITY TEST 濕度測試 | EXPOSURE TO 60±2℃, 90±5%RH(RELATIVE HUMIDITY) FOR 96 HOURS, THEN RETURN TO ORDINARY TEMPERATURE AND HUMIDITY FOR 2 HOURS. 在溫度60±2℃, RH90±5%(相對濕度)條件下放置96小時後在正常溫度條件下放置2小時 |
| 6-5 | HEAT CYCLE TEST 熱循環測試 | EXPOSURE TO HEAT TEST CONDITION UNDERLINE FOR 5 TIMES, THEN RETURN TO ORDINARY TEMPERATURE AND HUMIDITY FOR 24 HOURS. 在下圖測試條件下循環5次後在正常溫度和濕度條件下放置24小時 1 Cycle 1 Cycle 70 2OC 30 2OC 0 1 st 2ed 3 rd 4 th (Hour) |
| 6-6 | VIBRATION TEST 振動測試 | VIBRATE WITH 1.5 MM, 10HZ-55HZ-10HZ PER MINUTE, X-Y-Z EACH DIRECTION FOR 30 MINUTES AT STANDARD PACKING CONDITION. 正常包裝情況下, 每分鍾10HZ-55HZ-10HZ循環, 振幅1.5mm條件下沿XYZ三個方向各振動30分鐘 |
| 6-7 | DROP TEST 落體測試 | SET THE MOTOR TO THE APPROXIMATELY 100 G(INCLUDING THE MOTOR) WEIGH OF BLOCK, FREE DROP DIRECTION 3 TIME ON THE CONCRETE FOLLR 馬達連正常包裝約100g, 從1.5米的高度自由落落體到水泥地面, X-Y-Z三個方向各3次. |

| SPECIFICATION |  | 技術指導書 |  |  |
| --- | --- | --- | --- | --- |
| VIBRATION MOTOR | MODEL NO.: # FD-7BL5 |  | DATE: 2010/10/05 |  |
| 振動馬達 |  |  | PAGE: 5/6 | REV: A |

7. CAUTIONS 注意事項 :

  - ALLOWABLE RANGE FOR USE PAY ATTENTION TO VOLTAGE AND CURRENT RANGES

FOR USE, AND USE THE VIBRATION MOTOR IN THE CONDITION ACCORDANCE WITH THIS SPECIFICATIONS, OR THE LIFE AND PERFROMANCE SHOULD BE CONSIDERABLY REDUCED.

工作範圍

注意電壓及電流的使用範圍, 並且在本指標書規定情況下使用馬達, 否則將相應降低馬達的使用壽命和性能

  - STORAGE

STORAGE TEMPERATURE: -40℃ ~ + 85℃

STORAGE RALATIVE HUMIDITY: 10-90%

AVOIDS STORING IN HIGH TEMPERATURE, HIGH HUMIDITY OR CORROSIVE GAS ENVIRONMENT.

儲存

儲存溫度: -40℃ ~ + 85℃

儲存濕度: 10-90%

避免放置在高溫, 潮濕及有腐蝕性氣體的環境中

  - SOLDERING CONDITIONS

焊接溫度條件

250℃ --- WITHIN 5 SECONDS 五秒內

350℃ --- WITHIN 3 SECONDS 三秒內

7-4  HANDLING OF MOTOR

- TO HANDLE AND HOLD THE VIBRATION MOTOR CASE SOFTLY.

- DO NOT BRING A MAGNETIZED OBJECT NEAR OR CONTACT WITH THE SURFACE OF MOTOR TO AVOID OF PERFORMANCE BEING DETERIORATED.

- PAY ATTENTION MUST TO THE HANDLING AND WORKING ENVIROMENTS OF MOTOR, BECAUSE SUCH OBJECTS AS IRON POWDER IF ATTRACTED BY MOTOR MAGNET, WILL CAUSE NOISE CHARACTERISTIC DETERIORATION, THUS REDUCING THE RELIABILITY.

使用馬達

- 小心拿取, 使用

- 避免放置在磁性物體附近或接觸其表面, 這將影響其功效

- 注意操作環境, 尤其注意若鐵粉被磁鐵吸附, 將出現噪音問題, 降低使用壽命.

| SPECIFICATION |  | 技術指導書 |  |  |
| --- | --- | --- | --- | --- |
| VIBRATION MOTOR | MODEL NO.: # FD-7BL5 |  | DATE: 2010/10/05 |  |
| 振動馬達 |  |  | PAGE: 6/6 | REV: A |