| Chip Inductors |  |
| --- | --- |

Category; Feature; Type; Dimensions (mm); Inductance Range(H); Allowable DC Current (A); PDF. Chip; Thin film (High precision); KL73; 1E; 1.0 x 0.5; 0.56n 33n; 150m 700m. 1J; 1.6 x 0.8; 1.0n 68n; 100m 650m. 2A; 2.0 x 1.25; 1.0n 82n; 150m 900m. 2B; 3.2 x 1.6; 2.2n 100n; 200m 1.0. Air core (High Q, High current); KQC; 0402; 1.0 x 0.5; 1.4n 6.2n; 900m 1.4. 0403*; 1.0 x 0.75; 0.8n 21n; 600m 2.3. 0603; 1.6 x 1.05; 1.2n 27n; 1.0 2.25. Air core (High Q); KQ; T0402; 1.0 x 0.5; 1.0n 120n; 120m 1.36. 0603; 1.6 x 1.0; 1.6n 1.2μ; 80m 700m. 0805; 2.0 x 1.5; 3.3n 820n; 180m 600m. 1008; 2.5 x 2.2; 10n 10μ; 150m 1.0. Wire wound, Molded; LFC; 32; 3.2 x 2.5; 5.0n 330μ; 50m 450m.

|  |  |  | * Under development |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

|  | Power Inductors/Surface Mount |  |
| --- | --- | --- |

Category; Feature; Type; Dimensions (mm); Inductance Range(H); Allowable DC Current (A); PDF. SMD; Planar inductors (Ultra thin type); PL; 2520; 2.5 x 2.0 x 0.5; 0.33μ 4.7μ; 230m 1.1. 3225; 3.2 x 2.5 x 0.5; 1.1μ 5.1μ; 300m 800m. Power inductors (Non-magnetic shielded); LPC; 4545; 4.1 x 4.6 x 4.6; 1.0μ 1.5m; 100m 3.66. 4235; 4.5 x 4.2 x 3.5; 0.82μ 220μ; 200m 3.34. 4045; 4.5 x 4.0 x 4.5; 1.0μ 680μ; 120m 3.1. 9040N; φ10.2 x 4.9; 10μ 680μ; 140m 1.55. 9040E; φ10.2 x 4.9; 10μ 680μ; 350m 2.4. 10065; 10.4 x 10.0 x 7.5; 0.68μ 3.3m; 210m 9.5. 12065; 12.4 x 12.0 x 7.5; 0.68μ 6.8m; 160m 10.0. SDR; 0603; φ5.6 x 3.7; 1.5μ 82μ; 400m 3.0. 0604; φ5.6 x 4.5; 3.3μ 220μ; 350m 2.0. 0805; φ7.5 x 5.0; 10μ 470μ; 340m 2.3. 1006; φ9.5 x 5.5; 10μ 820μ; 240m 2.6. 0906; 13.5 x 12.7 x 6.3; 10μ 1.2m; 200m 2.1. Power inductors (Magnetic shielded); LKS; 0745; 7.5 x 7.5 x 4.5; 3.3μ 470μ; 400m 4.5. 1045; 10.1 x 10.1 x 4.5; 3.3μ 470μ; 500m 5.8. 1260; 12.5 x 12.5 x 6.0; 3.3μ 470μ; 700m 6.6. SDS; 0804; 10.5 x 8.0 x 3.7; 5.0μ 82μ; 400m 1.7. 0805; 10.5 x 8.0 x 4.5; 2.2μ 120μ; 450m 2.5. 1003; 12.7 x 10.3 x 2.7; 2.2μ 220μ; 260m 2.76. 1005; 12.7 x 10.0 x 4.9; 2.2μ 390μ; 330m 3.1. 0906; 12.7 x 10.5 x 6.0; 10μ 10m; 70m 1.8. 0908; 12.7 x 10.5 x 7.5; 10μ 15m; 75m 3.0. 1205; 12.7 x 12.7 x 5.0; 2.5μ 100μ; 1.0 5.0. 1206; 12.7 x 12.7 x 6.0; 2.5μ 820μ; 350m 6.2. 1208; 12.7 x 12.7 x 8.0; 2.5μ 1.0m; 500m 7.5.

|  | Common Mode Choke Coils/Surface Mount |  |
| --- | --- | --- |

Category; Feature; Type; Dimensions (mm); Inductance Range(H); Allowable DC Current (A); PDF. SMD; Common mode; SLF; 0905; 9.2 x 6.0 x 5.0; 10μ 2.0m; 600m 1.6.

|  | Transponder Coils |  |
| --- | --- | --- |

Category; Feature; Type; Dimensions (mm); Inductance Range(H); Unload Q 125kHz; PDF. SMD; LF Antenna (Receiving); KT; 11835; 11.8 x 3.6 x 3.0; 12m; 45.

Common Specification. Items; PDF. Minimum Ordered Quantity. Recommended Pad Dimensions. Packagings For Chip Components. Axial Tapings. Radial Taping. Forming. Color code•Resistance marking•E series numbers..