E27 Bulbs 11W LED SPOTLIGHT

Mode TYPE: JCA70CT-11WXR-E27

Product Specifications

Features:

- LED Photo source : CREE CHIP LED inside

- Bulb Type: Socket drop-n replacement halogen lamp

- Base Type: E26

- Beam Angle: 120 Degrees.

- Total Power: 11.0 Watt.

- Light Output: 850~1000Lm

- Color Temp: 2500K,2900K,4300K,5000K,6300K

- Estimated Lifespan: 20,000 hours

- Cool light, No UN or IR Radiation.

- CE & RoHS & LVD & EMC approvals.

- Optimized Thermal Management: Body temperature ＜65℃(Ta=25℃)

- Switching Power: high efficiency and constant current designs.

- Net Weight: 180g

Typical Applications:

- Landscape accent/Sport lighting

- Architectural lighting

- Display case accent

- Artwork lighting

- Mood lighting/General lighting

- Amusement park accent

Materials as Follows:

- Aluminum Cup: Skived heat sink.

- Optical Lens: Optical grade PC Lens

- LED Light Source: 0.5W X 20pcs

- Light Base: PC

- Power module: Switching Mode

- Connect Pin: Cu Plating Sn

Electrical Specifications:

- Operation Supply range: AC 100~240V (50Hz/60Hz)

- Input current: 110 mA

- Total power dissipation: 11W

- LED forward current: 300 mA

- Operation Specifications: -40℃ ~ +75℃

- Storage Specifications: -40℃ ~ +85℃