【產品規格 】

|  | 2.5” SATAII SSD with DRAM Buffer ( JMicron JMF612) | 2.5” SATAII SSD with DRAM Buffer (JMicron JMF616) | 2.5” SATAII SSD (SandForce SF-1222) |
| --- | --- | --- | --- |
| Interface | 7+15 pin SATA I/II | 7+15 pin SATA I/II | 7+15 pin SATA I/II |
| Capacities | 32GB~256GB | 32GB~256GB | 60GB~240GB |
| Transfer Mode | SATA-II 3.0Gb/s | SATA-II 3.0Gb/s | SATA-II 3.0Gb/s |
| R/W Performance (max.) | 240MB/s, 180MB/s | 250MB/s, 220MB/s | 285MB/s, 275MB/s |
| Random Write @ 4K | 30MB/s | 40MB/s | 80MB/s |
| TRIM support | Operating System support required | Operating System support required | Operating System support dependent |
| DRAM Cache | 64MB DRAM | 64MB DRAM | N/A |
| MTBF | >1,500,000 hours | >1,500,000 hours | >1,500,000 hours |
| Power Cycling | >3,000 Times | >3,000 Times | >3,000 Times |
| ECC Support | Built in BCH 16, 24-bit ECC | Built in BCH 16, 24-bit ECC | Built in BCH 16, 24-bit ECC |
| Power Consumption | DC 5V Operating 2W Idle 0.5W | DC 5V Operating 2W Idle 0.5W | DC 5V Operating 2W Idle 0.5W |
| Operating Temp. | 0°C~70°C | 0°C~70°C | 0°C~70°C |
| Storage Temp. | -40°C~+85°C | -40°C~+85°C | -40°C~+85°C |
| Operating Shock | 1,500G (@ 0.5msec half sine wave) | 1,500G (@ 0.5msec half sine wave) | 1,500G (@ 0.5msec half sine wave) |
| Vibration Resistant | 15G/10~2000Hz w/ 3 axis | 15G/10~2000Hz w/ 3 axis | 15G/10~2000Hz w/ 3 axis |
| Dimension (W x H x T) | 99.88x69.63x9.3mm | 99.88x69.63x9.3mm | 99.88x69.63x9.3mm |
| CE/FCC/VCCI | YES | YES | YES |
| RoHS Compliant | YES | YES | YES |
| Warranty | 2 Years | 2 Years | 2Years |

、